Chapter 2: Basic Information

2.1 Incremental forming technology

2.1.2 Development history

Incremental forming is a manufacturing method first proposed in Japan in the 1990’s. The goal of this manufacturing method is to make a dieless plastic processing method for low volume, complex items. Incremental forming is a general term for the processing of plastics without dies and includes various techniques such as hot forming and local heating. In general, Incremental forming methods have five main characteristics:

- High degree of freedom in moldable shapes

- Not requiring a mold

- Forms thin shell shapes

- High forming limit for metal sheets

- Creating complex shapes that are in low demand

2.1.3 Different types of incremental forming

There are two main types of incremental forming, dynamic and static (you need reference for this, a book about incremental forming or a journal that states about these two distinct types). Dynamic incremental forming, which uses kinetic energy to form the shape out of the material. An example of this is the shot peen method where millimeter sized shots made of various materials like steel, glass or ceramic, are shot at the material that you want to shape. Static incremental forming slowly stretches the material into the required shape. In this research paper, static incremental forming will be the method that we are using.

    - Uses of incremental forming

Incremental forming methods are useful for a variety of things like prototyping, creating custom parts for cars, and testing the characteristics of new materials. A real-world example of where incremental forming is used is in the LEXUS LC, EDGE produced by the Toyota Motor Corporation (add an image of these product and refer to them accordingly). The part of the car that uses incremental molding is in the plastic bumper at the front of this car.

2.2 NC machine tools​

    - Overview​

NC machines, or Numerical control machines, are machines that can be controlled with a set of numerical commands. The commands given to the machine can determine things like the position and speed of the tool. A series of operations related to machines can be executed by programmed commands. This allows machines to do what was previously done manually by humans to be automated, shortening processing times, improving productivity, and reducing labor costs.  In recent years, a control method called CNC, which is a form of computer-based numerical control, has become common.

An NC program is a program for automatically operating NC machines. A program consists of blocks, which are the individual lines like the one below:

G01 X8 Y10 Z9;

Blocks are generally broken up into two different parts, the Function section (G01) and the coordinate section (X8 Y10 Z9;). The NC program also can be programed to control the start and end of machining with specific notation specified in the table bellow (Table X). Depending on the function that you want, the function section (G01) will change. The following are general descriptions for the different types of functions.

- G-Code

  - General Functions become preparatory functions in G-code and NC programs

- M-Code

  - Miscellaneous Functions are M-codes, auxiliary functions, and are primarily used for commands other than movement.

- F-code

  - Control the F-code and feed speed from Feed Functions.

- S-Code

  - S-code controls the spindle rotation speed.

The following table has the functions that are used in the G-code during this research.

Table 2.1: G-code functions

| G Code | Positioning |  | G00 |
| --- | --- | --- | --- |
|  | Linear interpolation |  | G01 |
|  | Circular interpolation | Clockwise | G02 |
|  | Helical interpolation | Counter clockwise | G03 |
|  | Plane Selection | XY Plane | G17 |
|  |  | YZ Plane | G18 |
|  |  | ZX Plane | G19 |
|  | Inch input |  | G20 |
|  | mm input |  | G21 |
|  | Local coordinate system settings |  | G52 |
|  | Machine coordinate system selection |  | G53 |
|  | Work coordinate system selection |  | G54-G59 |
|  | Absolute coordinate command |  | G90 |
|  | Relative coordinate command |  | G91 |
|  | Work coordinate system settings |  | G92 |
| M Code | Spindle section |  | M03 |
|  | Program end |  | M30 |
| F Code | Feed Speed (mm/min) |  | F5 |
| S Code | Rotation Speed (RPM) |  | S5 |

2.2.2 3D CAD

CAD (Computer Aided Design) is software for designing on a computer. CAD can be broadly divided into two types: 2D CAD and 3D CAD. With improvements in technology, 3D CAD has been widely adopted in various industries. 3D CAD design software allows for easy planning of prototypes so there are less errors in the production of the products. All test pieces and molds in this research paper were designed using CAD to be manufactured and tested.

2.2.3 CAM

CAM (Computer Aided Manufacturing) is software that creates machining programs for NC machine tools on a computer. Before CAM, workers had to input NC data directly into the machine tools, increasing the time for manufacturing and costs. As mentioned in the G-code section, CAM eliminated this process, speeding up the manufacturing process. In addition, because it is possible to visually verify the machining process through machining simulation, it allows the manufacturing to be even more efficient.

    - Tool path

A tool path is the path along which a machining tool moves. A tool path is created using the 3D coordinates of an STL file from CAD software to obtain a collection of all the points or point cloud from the triangles of the STL file. From this you sort the points, delete the duplicate points, generate a set of coordinates, then add the necessary pieces so that the coordinates turn into the NC program. The NC program can be used to instruct the machining path of the tool. The toolpath was viewed using a web browser called NC viewer to identify any potential issues before any manufacturing was done.

    - Python

Python is a programming language that is widely used and known for its simplicity, readability, and versatility. Python allows developers to condense programs in fewer lines of code compared to other languages like C++ or Java. In this research, Python was the programming language of choice to develop the CAM system used to produce the toolpath for its easy understanding and application. The code for the CAM system will be explained in a later section.

2.3 CFRP (Carbon Fiber Reinforced Plastic)

    - Overview

Carbon fiber Reinforced plastics or CFRP, is a general term for plastics that have been reinforced with carbon fibers. The reinforcement allows CFRP to have high strength and rigidity compared to metal materials such as iron and aluminum but also being significantly lighter. It also has excellent design and vibration absorption capabilities. As such, there has been an increase in the use of CFRP in many things like aircraft wings, car parts, sports equipment, and even medical uses.

2.3.2 Classification of CFRP

CFRP is classified into two types, Thermosetting CFRP (CFRTS: Carbon Fiber Reinforced Thermo Sets) and Thermoplastic CFRP (Carbon Fiber Reinforced Thermoplastic) CFRTS’s are typically used for weight reduction in sports equipent, aircraft, etc. CFRTS has the following characteristics:

- High specific strength, high heat resistance

- Long molding time, high cost

- Not possible to recycle

CFRTP is also used in similar situations to CFRTS and has already been used in some automobile parts which will be discussed in a later section. CFRTP has the following characteristics:

- Specific strength and heat resistance are inferior to CFRTS

- Short molding time, low cost

- Partially recyclable

CFRP can be also divided into two other classifications depending on which direction the plastics in the CFRP are oriented. The two sections, continuous fiber and discontinuous fiber have the following characteristics.

Continuous fiber CFRTP:

- Layered structure

- High specific strength

- Low press formability

Discontinuous fiber CFRTP

- Integrated structure, random orientation

- Specific strength is inferior to long fibers

- High press formability

2.3.3 CFRP application areas

As mentioned briefly in the previous section, CFRP can be used in various fields. In aircraft and car parts, CFRP is used in conjunction with other materials to make more efficient parts. Things like airplane wings and car parts like bumpers. CFRP can also be used in the medical field in things like head braces for brain surgery to allow x-rays of the brain to be taken while keeping the patients head stable, to a socket for prosthetics, taking advantage of its light weight to reduce the overall weight of prosthetics.

2.3.4 CFRP sample used in this study

In this study, discontinuous fiber CFRTP is used for its short molding time, low cost, and its recyclability.

2.4 Previous research A (Incremental forming of CFRTP under thermal bath conditions using oil bath and the creation of a CAM system using Python language.)

    - Overview

The goal of this previous thesis is to create a CAM system using python to perform incremental forming for discontinuous fiber-reinforced thermoplastics (CFRTP) using an oil bath to heat the CFRTP to enable incremental forming without molds. The previous researcher developed a custom CAM system in Python to generate G-code tool paths from CAD models, targeting low-volume, high-mix production needs. The python code can be seen in the next section.

The study addresses challenges with traditional manufacturing like high mold costs and limited flexibility, especially in forming thin shell components. It introduces a cost-effective alternative that combines material heating via an oil bath and open-source digital toolpath generation.

    - CAM and CAD system

The CAM system that was created achieves three goals, to read STL files from Creo Parametric 4.0., Extract 3D point clouds of target shapes, and create a tool path in G-code. The code achieves this by using pythons’ libraries as seen in figure X to create a point cloud from the STL file, converts the clouds into layers by Z-height then connects the points in each layer by the minimum Euclidean distance. Finally, it takes the coordinates and adds the proper syntax to be used by the CNC machines.

The Shape that was created in Creo was a truncated cone with varying dimensions. The radius of the shape ranges from 20, 30, and 40 mm, and similarly the wall angle 30°, 40°, and 50°, and the pitch or the resolution of the STL was 1. The dimensions were selected to match the working area and limitations of the experimental setup. The shape was chosen because it is a good shape for a proof of concept, especially for plastic deformation layering.

import pyvista as pv

import csv

import pandas as pd

import numpy as np

mesh = pv.read('test_piece_3.stl')  # Load the STL file and name it "mesh"

points = mesh.points  # Get the point cloud from "mesh"

with open('dots.csv', 'w') as f:  # Create a new CSV file

writer = csv.writer(f)  # Set to write mode

writer.writerows(points)  # Write the point cloud

df = pd.read_csv('dots.csv')  # Open the CSV file with pandas

df = df.dropna()  # Remove blank rows

df = df.set_axis(['x', 'y', 'z'], axis='columns')  # Label columns as x, y, z (with space for row numbers)

maxi = df['z'].max()

mini = df['z'].min()

df = df[df['z'] > mini + 0.01]  # Remove the bottom plane

df.to_csv('dots.csv')  # Overwrite the CSV file

df = pd.read_csv('dots.csv')  # Re-open the CSV file with pandas

ll = [0, 0, 0]  # Specify that this is a 3-dimensional numeric array

pp = [0, 0, 200]  # Set a hypothetical initial tool position

Figure 2.1: Initial setup

while len(df) > 0:

s = df['z'].max()  # Find the maximum value along the z-axis

df_z = df[df['z'] == s]

df_z.to_csv('layer.csv')

df_z = pd.read_csv('layer.csv')

df_z = df_z.drop(df_z.columns[[0, 1]], axis=1)

df_z.to_csv('layer.csv')

p = df_z['z'].idxmax()  # Maximum value on the z-axis, starting from this point for processing

l = np.loadtxt('layer.csv', delimiter=',', dtype='float64', skiprows=1, usecols=[1, 2, 3])  # Convert from CSV to ndarray

n = len(df_z)  # Get total number in df

c = 0

if n == 1:

pp = l

ltp = [0, 0, 0]  # Tool path design order

if n > 1:

while n > 0:

j = 0

ld = []

if c == 3:

l = np.vstack([l, llk])

n = n + 1

while j < n:

ld.append(np.linalg.norm(pp - l[j]))  # Calculate and list distances from p

j += 1

k = ld.index(min(ld))  # Index of the smallest value in ld

lk = l[k]  # Array at the index of the smallest value in l

ltp = np.vstack([ltp, lk])  # Add lk vertically as an array to ltp

if c == 0:

llk = lk

ld.pop(k)  # Delete the k-th element in ld

l = np.delete(l, k, 0)  # Delete the k-th row in l

n -= 1  # Reduce n by one

pp = lk  # Set next p as the current k

c += 1

ltp = np.delete(ltp, 0, 0)

ll = np.vstack([ll, ltp])

else:

ltp = np.delete(ltp, 0, 0)

ll = np.vstack([ll, l]) #Transform df to remaining layers

df=df[df['z']<s]

ll = np.round(ll, 3)  # Round to 3 significant digits

with open('toolpath.csv', 'w') as f:

writer = csv.writer(f)

writer.writerows(ll)  # Save ll to CSV

Figure 2.2: Toolpath generation

df_ltp = pd.read_csv('toolpath.csv')

df_ltp = df_ltp.set_axis(['x', 'y', 'z'], axis='columns')

df_ltp.to_csv('toolpath.csv')  # Convert to G-code from here

l = np.loadtxt('toolpath.csv', delimiter=',', dtype='float64', skiprows=1, usecols=[1, 2, 3])

lx = l[:, 0]  # 0th column of l

ly = l[:, 1]  # 1st column of l

lz = l[:, 2]  # 2nd column of l

map1 = map(str, lx)

lx = list(map1)  # Change data type to string

map2 = map(str, ly)

ly = list(map2)  # Change data type to string

map3 = map(str, lz)

lz = list(map3)  # Change data type to string

n = len(df_ltp)

for i in range(n):

lx[i] = ('X' + lx[i])  # Add "X" prefix to n coordinates

for i in range(n):

ly[i] = ('Y' + ly[i])  # Add "Y" prefix to n coordinates

for i in range(n):

lz[i] = ('Z' + lz[i])  # Add "Z" prefix to n coordinates

df_gchord = pd.DataFrame(data={'G': 'G01', 'x': lx, 'y': ly, 'z': lz, 'EOB': ';'})

glist = df_gchord.values

outf = open("Gchord.csv", 'w', newline='')

writer = csv.writer(outf)

writer.writerows(glist)

outf.close()

df_g = pd.read_csv('Gchord.csv')

df_g.to_csv('Gchord.txt', index=False, sep=" ")

Figure 2.3 G-code conversion and exporting the G-code to a text file.

2.4.3 Experimental equipment:

In the previous research the following equipment were used, most of the materials used in the previous experiment will be used in the research and will be explained more in detail in a later section.

- CFRTP sheet (PA6 matrix, 20 wt% carbon fiber)

  - Size: 110 mm × 110 mm × 1 mm

  - Dried at 120°C for 24 hours using an ETTAS oven to prevent moisture-induced degradation.

- Heating Equipment:

  - Oil bath: T-22H model (Thomas Scientific)

    - Temperature range: up to 200°C

    - Precision: ±0.2°C (PID controlled)

    - Stirring method: jet circulation

- Oil used: KF968 (Shin-Etsu)

  - Non-volatile, stable to 250°C, fire-resistant

- Forming System:

  - CNC Machine: Sodick MC430L

    - Travel: X 420 / Y 350 / Z 200 mm

    - Feed: up to 36 m/min

  - Tool (Mandrel): SUS304, 5 mm dia, 2.5 mm radius tip, 60 mm long

  - Custom jig submerged in the oil bath with secure clamping and easy detachment for safety and repeatability.

2.4.4 Results

From the experiments that were done, it was found that there were several factors that contributed to a successful forming which were, the temperature of the material, the tool rotation, wall angle, radius variations. The temperature that was achieved the best results ranged from 180–200°C (453–473 K) and the lowest temperature that had a successful forming was 323 K. Below 160°C (433 K), material failed to deform properly, often leading to cracks.

It was also found that the rotation of the tool at 1000 rpm played a big role in increased depth and smoother walls. When there was no rotation the finish on the surfaces were poor and tended to have uneven wall thickness.

The angle of the wall had some influences on the forming. The best wall angles tended to be both 30° and 40°. At these angles, it was found that there was smooth forming, minimal springback and when it was closer to 40° there was some deformations that were observed. Anything that was 50° or greater had risks of cracks and breakage.

The radius of the shape also played a big part in forming, it was found that the smaller the radius (20 mm) there was a deeper forming depth but a much higher wall stress, and 40 mm produced less depth but more uniform wall strength.

The maximum and minimum criteria for a successful forming was the following:

  - Maximum: 14.9 mm at 200°C, 20 mm radius, 40° wall angle.

  - Minimum: 3.6 mm at 160°C, 50° wall angle, without rotation.

2.4.5 Conclusions:

The researcher successfully created the Python CAM System and was able to successfully generated tool paths from STL data, that also smoothed the process of incremental forming, improved the flexibility of the existing toolpath generation methods, and allowed the support arbitrary shapes, ultimately creating a system that could be used in production environments.

The CAM system that was created in this previous research A was used as a starting point for the CAM system that will be shown later, there are a lot of similar aspects but also some improvements from what is shown here.