Slide 0:

Hello. My name is Karter Wong, and I will present my research topic: the Development of CAM systems for the Incremental Forming of CFRTP from 3D CAD data.

Slide 1:

Today, I will briefly review some background information, then previous research on this topic, the progress I have made since the last presentation, and the plan for the remaining part of the semester.

Slide 3:

Incremental forming

Incremental forming is a flexible forming process that can shape sheets of various materials using plastic deformation of the sheet with a tool into the desired shape.

Unlike traditional manufacturing processes, it does not require dies or molds, making it ideal for low-volume production, prototyping, or custom parts. The tool follows a programmed path to manufacture thin shell shapes, offering reduced setup and production costs.

It also allows producing various products in small quantities, where mold creation would be too expensive.

Slide 4:

An example of Incremental forming in use today is by car manufacturers like Ford, Nissan, and Lexus. These manufacturers use incremental forming to prototype new vehicle designs and add minor customizations to vehicles, as seen on the Lexus LS edge, as shown in the red circle of the figure on the right.

Slide 5:

Previous research on this topic developed a simple CAM system to generate a tool path and conducted experiments to test the tool path. The experimental conditions of the experiments will be described later.

The CAM system developed during the previous research using Python can be described in four steps:

First, STL data from the 3D CAD data is converted into a point cloud, a collection of points from the triangles given by the STL file.

Next, the point cloud's points are grouped into layers defined by the Z coordinate. During this process, the program looks for the points on the outermost part of the figure so that the toolpath does not go in the middle of the forming shape.

The third step is to construct the toolpath, which is done by calculating the distances from the tool's current position to the other points on the layer and moving to the point with the shortest distance. This process is repeated for all the points on a layer and each layer.

The final step is to convert the X, Y, and Z coordinates into G-code so that the machine can read the generated tool path.

Slide 6:

Using the previous research as a base, I have added some changes to the CAM system to work with complex shapes. These include the initial and ending commands needed for every toolpath, Convex hull, reverse offset, and other smaller features to make the program's transition from designing to machining smoother.

In the updated CAM system, instead of just loading the STL file as it is and forming the point cloud, the CAM system rounds all the Z coordinates to the nearest whole number. It then forms the point cloud but also sets a uniform layer distance of 1 mm and, like the other program, goes from the highest layer to the lowest.

Next, the CAM system finds the outer boundary of points using the convex hull library while removing layers with less than 5 points. Then, the points are filtered to remove points within a certain radius of the current point. Then, the CAM system sorts the points again so that the toolpath is smooth and the points are all "visited" by the tool once. Then, the program closes the layer by connecting the last point to the first point.

After generating the toolpath, the X, Y, and Z coordinates are rounded to increase the accuracy of the tool paths. The reverse offset is applied, shrinking the toolpath by the diameter of the tool.

Finally, the coordinates are converted to G-code by adding the proper syntax, initial setup commands like the spindle speed, feed rate, etc., and the program's termination sequence, which moves the toolpath to a safe height and stops the machine.

These changes allow the CAM system to generate a smooth toolpath for complex shapes, optimize the generation process, and allow the operator to start machining immediately after the toolpath is generated without any adjustments.

Slide 7:

Here is a comparison of the CAM system used in previous research and the updated CAM system for the current test shape, which is more complex than the previous research test shape, which was a truncated cone.

The updated CAM system produces a simplified tool path, as seen in the figure on the left. The Previous CAM system processed all the points from the point cloud, taking over an hour and a half to generate the toolpath versus the updated system's 2 seconds.

Slide 8:

Since the last interim presentation, the test shape was changed from the figure on the right to the figure on the left due to the 90-degree angles and details that were too pointy.

The updated shape is a simpler version of the previous test shape and is shorter; the dimensions of the current test shape are 30 mm x 30 mm x 9.9 mm. These dimensions of the figure were chosen to ensure the figure would not have any 90-degree angles while keeping some of its complexity.

Slide 9:

To conduct the experiments, I used an MC430L machining center on nylon sheets that were about 110 mm x 110 mm x 1mm, with a mold of 89.92 mm x 89.92 mm x 20 mm of my test shape, which is 30 mm x 30 mm x 9.9 mm. The tool that was used is a 2.5 mm diameter rod.

Slide 10:

The test shape was successfully formed with the mold during the tests. The general shape was machined; however, in some areas of the shape, there were thinner portions. The thin parts of the shape were due to the tool path being too close to the mold below.

All the tests were done with a feed rate, or processing speed, of 200-400 mm/min, and the spindle was spinning at 250 to 300 RPM.

Slide 11:

For the tests without the mold, the material was visibly thicker after the machining but could not reach the desired depth. This was due to the material's springback. Lowering the tool's origin addressed and fixed this.

All the tests were done with a federate, or speed of processing, of 200-1000 mm/min, and the spindle was spinning at 250 to 500 RPM.

For the experiments, it was harder to form without the mold as the spring back would be higher. This resulted in needing a higher feed rate and spindle speed so the nylon would stay in the forming shape.

Slide 12:

Between now and the next presentation, my goal is to conduct experiments with CRTP in an oil bath with and without the mold. I will also learn how to use the 3D scanner to analyze the results, as right now, it is hard to determine accurately if the test piece was formed to the correct specifications.

After this, I will attempt to improve the formability of the test shape.

The experimental equipment will be the same as that used in previous research. The T -22H oil bath and silicon oil will be used to heat the CFRTP for forming.

The test piece will be a sheet of CFRTP that is 110 mm × 110 mm thick and 1 mm thick.

Slide 13:

The experimental conditions used for the oil testing will be based on the previous research's experimental conditions for the experiment that had the best results, as shown in the table on the right. The feed rate, spindle rotation, pitch, and temperature will be the same, while the other conditions will differ as the test shape is different.

Slide 14:

Thank you for listening. Do you have any questions?