%pip install pyvista

%pip install pandas

%pip install scipy

%pip install scikit-learn

'''From here to the next section is setting up the variables for the rest of the code'''

import pyvista as pv

import csv

import pandas as pd

import numpy as np

from scipy.spatial import ConvexHull

from scipy.spatial.distance import pdist, squareform

from scipy.optimize import linear_sum_assignment

pd.options.mode.copy_on_write = True

# Load the STL and converting the dots into a CSV file that python can edit

mesh = pv.read('test_piece_3.stl')

points = mesh.points

with open('dots.csv', 'w') as f:

writer = csv.writer(f)

writer.writerows(points)

# Sorting the dots.csv file that was made in the previous step in order to define coordinate points (X,Y,Z)

df = pd.read_csv('dots.csv').dropna()

df = df.set_axis(['x', 'y', 'z'], axis='columns')

# This is removes the bottom layer of the shape in order to prevent the machine from going back over the already processed points

df = df[df['z'] > df['z'].min() + 0.1]

# Setting variables for the rest of the code, specifying the number of layers wanted

num_layers = 175  # Specify the desired number of layers

# The computatoin for ensuring that the Z-coordinates or "layers" have the same spacing between each other

z_min, z_max = df['z'].min(), df['z'].max()

layer_spacing = (z_max - z_min) / num_layers

# This groups the Z coordinates so that the layers are defined

df['z'] = np.round((df['z'] - z_min) / layer_spacing) * layer_spacing + z_min

# This section is used to sort the points using TSP to ensure that there is no long jumps in the tool path and that the tool path stays 'clean'. This is just used to define a custom function.

def solve_tsp(points):

# Compute pairwise distances

distance_matrix = squareform(pdist(points))

# Solve TSP using the Hungarian algorithm

row_ind, col_ind = linear_sum_assignment(distance_matrix)

return col_ind

# This section is used to set the points to be processed in the same way for every layer ensuring that the tool path stays 'clean'. This is just used to define a custom function.

def sort_clockwise(points):

center = points.mean(axis=0)  # Find centroid of the points

angles = np.arctan2(points[:, 1] - center[1], points[:, 0] - center[0])  # Calculate angles

return points[np.argsort(angles)]  # Sort points by angle

'''Creating the Tool path for the machine to process'''

# Initialize variables

ll = []  # This stores the tool path so that it can be saved to a txt file later

pp = np.array([0, 0, 400])  # This defines a starting point for the program to start processing from, this can be adjusted but changing this shouldnt matter to how the tool path looks.

while len(df) > 0:

# This identifys the current layer and identifies the greatest Z coordinate or highest layer.

current_layer_z = df['z'].max()

current_layer = df[df['z'] == current_layer_z]

# This layer skips any layers with fewer than 3 points because ConvexHull is unable to process layers with less than 3 points. This stops the program from breaking.

if len(current_layer) < 3:

df = df[df['z'] < current_layer_z]

continue

# Filter points to retain only boundary points using ConvexHull. This ensures that only the outermost points are processed and saved to the tool path.

layer_points = current_layer[['x', 'y']].values

hull = ConvexHull(layer_points)

hull_points = current_layer.iloc[hull.vertices]

# The next three lines are for using TSP to process the hull points

hull_coords = hull_points[['x', 'y']].values

tsp_order = solve_tsp(hull_coords)

tsp_sorted_points = hull_coords[tsp_order]

# This line is used to ensure that the hull points are sorted and processed in a clockwise direction.

sorted_hull_points = sort_clockwise(tsp_sorted_points)

# Rebuild the sorted layer's coordinates to add the Z coordinate becase ConvexHull only sorts the X and Y coordinates

current_coords = np.hstack([

sorted_hull_points,  # X and Y coordinates

np.full((sorted_hull_points.shape[0], 1), current_layer_z)  # Z coordinate

])

# Process points in the current layer

layer_path = list(current_coords)

# This part of the code makes the tool path return to the first point of the layer and then adds this movement to the end of the tool path.

layer_path.append(layer_path[0])  # Return to the first point

pp = layer_path[0]  # Update tool position

ll.extend(layer_path)

# Remove processed points from the main DataFrame then from here it repeats untill all of the layers are processed

df = df[df['z'] < current_layer_z]

'''Bellow is converting the tool path into G-Code so that it can be used for actual machining'''

# Save tool path to a CSV file to convert the tool path to G-code

df_ltp = pd.DataFrame(ll, columns=['x', 'y', 'z'])

df_ltp.to_csv('toolpath.csv', index=False)

# Convert to G-code and save as a text file. (Can be changed to CSV or any file type.)

df_gchord = pd.DataFrame({

'G': 'G01',

'x': 'X' + df_ltp['x'].astype(str),

'y': 'Y' + df_ltp['y'].astype(str),

'z': 'Z' + df_ltp['z'].astype(str),

'EOB': ';'

})

df_gchord.to_csv('Gchord.txt', index=False, sep=" ", header=False)